# An ontological engineering approach towards combining Data Models and Information Models for IoT Semantic Interoperability

Position Paper for IoT Semantic Interoperability Workshop 2016

Xiao Ma, University of Warwick, x.ma@warwick.ac.uk

Jon Crowcroft, University of Cambridge, Jon.Crowcroft@cl.cam.ac.uk

Irene Ng, University of Warwick, Irene.ng@warwick.ac.uk

The rapid expansion of Internet of Things (IoT) development has led to interoperability requirement at various levels. Organisations have devoted a great amount of interoperability development towards M2M (Machine to Machine) in standardising protocols (data models) to allow relatively mature objects level interoperability.  Yet, application oriented information models interoperability is to be matured. Furthermore, interoperability between data models and information models still requires substantial development. This position paper proposes a novel approach to achieve IoT semantic interoperability with an attempt to provide answer to some of the issues specified for the workshop 2016 (https://www.iab.org/activities/workshops/iotsi/).

We propose an ontological engineering approach to combine Data Models and Information Models for IoT Semantic Interoperability. Here we propose considering both Data Models (DM) and Information Models (IM) as different forms as ontology (not in its philosophy definition but in information science practice), which represents / describes objects or applications through the definition of the entities, properties, and interrelationships of the entities (communication protocols as a special type of relationship). However, in order to cater the differences maturity levels of interoperability in DM and IM, we suggests existing Data Models as Taxonomies (defined as formal representations / structured language of detailed object description), such as the device / sensor types, components, associated properties, their actual data fields, formal language used in communication protocols, the syntax, relationship between syntax and the nested structure. For example, an object combining a temperature sensor, a motion sensor, and a light sensor may have a DM that centres on the object serial number, with each sensor’s serial number as a child node of the object; each child node will subsequently have its own children such as sensor type (from an existing type schema / standard), unit of measurement, communication protocols between a sensor and the object, and the numerical value of each measurement. This setting recognised the more consensus nature of DMs’ (DM audience are likely to be domain experts on IOT objects and communications), hence promotes them as taxonomies to form the foundation of the combined ontology. Existing Information Models are considered as Folksonomies (defined as relatively informal categorisation of objects derived from applications), such as more abstractive description (in natural languages) of a functionality, objects (and configuration) required in this functionality, schema of information generated, their relationships, and configurations of the information. For example, the same object may have an IM that still centres on the object but described simply as “the combined sensor xyz near the front door”, with child notes such as its physical location, time tag (may obtain a time tag from a different object such as a central hub) for all objects. Instead of actual readings of temperature, lights and motions, the IM may in fact abstract them as information that facilitate easier processing on the application side, such as warm/cold, bright/dark, and motion detected / no motion. This accepts the diversification of IM audiences and their use of natural language descriptions for applications in their own fashion, hence allows the variation of IMs as folksonomies to be add on to the combined ontological foundation.

With such configuration, we takes the principles of SENSUS ontology engineering methodology (Swartout, Patil, Knight, & Russ, 1997) to enable a construction of ontological network combining formal ontologies (in this case, combining DM taxonomies). This method firstly identifies a few key shared representations as the foundational nodes of the taxonomy, then adding other representations that connects to those key shared representation as child notes with its source structure, and how it should be interoperated with at least one existing DM (so it can be added as a child note). Therefore, we believe such “mapping of existing data models against each other can offer interworking”.

In addition, we further leverages an improved and extended methodology (Ma, Bal, & Issa, 2014) to expand SENSUS capability towards folksonomy inclusion, through extracting semantically related concepts and relationships to enrich IM descriptions and provide broader connections back to DMs. The expansion towards folksonomy will not only be executed against known IMs, but more widely against information schemas that have been made available online such as DBPedia (a crowd-sourced community effort to extract structured information from Wikipedia http://wiki.dbpedia.org/about), Schema.org (a collaborative, community activity with a mission to create, maintain, and promote schemas for structured data on the Internet, on web pages, in email messages http://schema.org/), and even semi-structured data sources such as dictionaries (i.e. WordNet https://wordnet.princeton.edu/). Such “online repositories for sharing schemas” will provide the width and breadth required to capture IM variations. In a similar manner, new IMs will be added to the existing ontological network as a new node that connects to an existing node, with its interoperability defined with the existing node. Therefore, we propose the “harmonization” can be achieved through such ontological network construction. As all neighbouring nodes will be interoperable with each other, the network will inherently allow any two targeting nodes can be interoperable via a range of other notes that connects the target nodes, even “the use cases of different groups and organizations are so unique”, that they may only be interoperable with just one existing node.

Our ontological engineering approach presents the advantage of aligning and merging a wide range of IMs and DMs with little domain expert input, and a matured suite of analytical methods for network analysis (such as concept and clustering identification, ontology pruning, and faceted viewing). This approach has been generalised as an algorithm and automated in a computational programme, so that we can leverage route optimisation techniques to build interoperability between two remotely connected nodes.

We anticipate the implementation of this approach in IoT semantic interoperability will provide an ontological network to map the DMs and IMs with enriched relationship to reflect “the state of the art in data and information models”. The expansion towards folksonomies will include IMs from “different organizations, companies and individuals develop extensions”. However, we expect a different fashion of interoperability among such IMs. It is because IMs may add their own facets (application driven perspectives) and dimensions (IMs that may be generalised) to further enriched ontological network, and the derived multi-faceted and multi-dimensional network can render asymmetry interoperability. In other words, IM A may not be interoperable with B via an alignment, but a faceted representation by clustering a few B’s sub-concepts with facet specific configuration. Therefore, we expect that “What should an information model look like” will be facet (or application) dependant, and even use case dependant. Namely, even an application specific IM may look differently when it is applied in different use context.

Consequently, we further argue that an ontological network can enable IoT Semantic Interoperability with possibly broader coverage of objects and IMs. However, a truly semantic interoperability will not only require IMs and DMs, but Use case context Models UMs to facilitate the IMs to gain the facet to render better interpretability from related IMs and DMs. With this viewpoint, “organizations better work together to increase awareness and information sharing” may not be necessary. Instead, as long as each organisation defines their DM or IM in the ontological network, with a Unique Identifier for each model, semantic interoperability can be realised.

Reference:

Ma, X., Bal, J., & Issa, A. (2014). A fast and economic ontology engineering approach towards improving capability matching: Application to an online engineering collaborative platform. Computers in Industry, 65(9), 1264-1275. doi: http://dx.doi.org/10.1016/j.compind.2014.05.004

Swartout, B., Patil, R., Knight, K., & Russ, T. (1997). Toward distributed use of large-scale ontologies. Paper presented at the AAAI'97 Spring Symposium on Ontological Engineering, Stanford University, California.